House electrical circuits

[image: image1.bmp]
The main diagram shows an example of the electrical wiring of a house. You can see that the fuse box contains different fuses for different circuits, the largest fuses being for those that take the most current.

The 13 A sockets are wired to the RING MAIN. This is a circular live wire and a circular neutral wire that loop round the house with the sockets connected between them.

The lights are wired in parallel with the switches on the live side of the lamps.

A two-way switch is shown, you will probably have one of these for your hall and landing lights. Either of the switches A or B will control the light.


The second diagram shows a 13 A plug. Notice that the length of bare wire in the plug is as short as possible and that the cable is held by a grip. The length of the earth pin is bigger than the others so that it will open the safety flaps in the socket as the plug is pushed in.
fuse





cable grip





neutral - blue





live - brown





earth – green/yellow





Ring main circuit





Main switch





Fuse box





5A





cooker





5A





15A





30A





30A





Immersion heater





Lighting circuit





13A socket





two-way switches




















