Mutual inductance

When the current in a coil is changing an e.m.f will be induced in a nearby circuit due to some of the magnetic flux produced by the first circuit linking the second. The phenomenon is known as mutual induction. It is important to realise that the induced e.m.f. lasts only as long as the current in the first circuit is changing.

Mutual inductance (M) = -E/[dI/dt]

The mutual inductance M is defined by the equation

where E is the e.m.f induced in the secondary coil and dI/dt the rate of change of current in the primary.

Two coils are said to have a mutual inductance of 1 H if an e.m.f. of 1 V is induced in the secondary when the current in the primary changes at the rate of 1 A s-1.

Induction coils such as this are used in car ignition circuits, and used to be a source of high voltage for research.

The long solenoid

# x

N1

N2

Consider the mutual inductance of a long solenoid and a coil as shown in the diagram.

Suppose that a short coil of N2 turns is wound round a solenoid of N1 turns, with a cross-sectional area A, length x and carrying a current I The flux at the centre of the solenoid is:

B = oN1I/x

The flux linking the short coil is  = BA and therefore the flux linkage of the short coil is

N2 =BAN2=  [oN1I/x]AN2

If the current in the primary changes by dI in time dt, giving a change in flux linkage of d(N) in the secondary, then the e.m.f. induced in the secondary will be

E = d(N)/dt = [oN1/x]AN2 dI/dt

Writing M as the mutual inductance, we have that   E = - MdI/dt   and therefore

Mutual inductance (M) = oAN1N2/x

Example

Calculate the mutual inductance of a pair of coils if the primary has 1000 turns of radius 2 cm and is 1 m long while the secondary has 1200 turns and is wound round the centre of the primary.

M = 4π x 10-7x 4 x 10-4 x 1000 x 1200

= 1.90x10-3 H = 1.90 mH